CoreConduit Build Plan

Project SIGNAL

Offline Survival InfoHub

A turnkey, low-power Raspberry Pi captive-portal access point that serves a curated knowledge library when the grid is down. This document is the build plan handed to Claude Code as the engineering specification.

Owner: Cory Potter, CoreConduit    Status: Plan v1.0 — ready for execution    Target: Raspberry Pi Zero 2 W / Pi 4

# 1. Project Objectives & Scope

SIGNAL is a self-contained Wi-Fi access point that hosts a captive-portal library of survival knowledge. Anyone in range connects with a phone or laptop, gets auto-redirected to a local landing page, and browses Wikipedia, medical references, repair guides, maps, and literature — all without internet.

### In scope

- Raspberry Pi configured as a Wi-Fi access point (no internet uplink required).

- Captive-portal DNS redirection so connecting clients are auto-routed to the local site.

- Kiwix-served ZIM archives (Wikipedia, WikiHow, medical references, OpenStreetMap).

- Mobile-friendly landing page in CoreConduit visual language.

- Automated provisioning: a single `install.sh` brings a fresh Pi OS Lite image to a working hub.

- Battery + status monitoring surfaced on the landing page (voltage, uptime, free storage).

### Out of scope (v1.0)

- Internet pass-through, user accounts, write-back features, mesh networking.

- Hardware design of the enclosure (covered separately by a 3D-print drop).

### Non-negotiable invariants

- No data exfiltration. Device must not initiate outbound connections in production mode.

- No phone-home. No telemetry, analytics, or update checks at runtime.

- Read-only content. Clients can only read. No upload, no comment, no write paths.

- Open source, MIT licensed. Consistent with CoreConduit tooling philosophy.

# 2. System Architecture

Four loosely-coupled layers. Each layer is replaceable; failure in one (e.g. Kiwix) should not crash another (e.g. the AP itself).

### Layer overview

| Layer | Component | Purpose |
| --- | --- | --- |
| Network | hostapd + dnsmasq | Broadcast SSID, assign IPs, wildcard DNS |
| Captive portal | nginx + iptables | Detect probes, force redirect to landing |
| Content | kiwix-serve | Serve ZIM archives on localhost:8080 |
| Frontend | Static HTML + small FastAPI status endpoint | Landing page, library directory, health |

### Request flow

- Client phone joins SSID `SIGNAL_INFOHUB`.

- dnsmasq assigns IP via DHCP and answers every DNS query with the Pi's gateway IP (192.168.4.1).

- Phone's captive-portal detector hits a known URL (e.g. `connectivitycheck.gstatic.com`); nginx returns a 302 to `http://hub.local/`.

- Landing page loads from nginx, listing categories: Wikipedia, WikiHow, Medicine, Maps, Library, Status.

- Selecting a library proxies through nginx to `kiwix-serve` on port 8080. Status pulls from FastAPI on port 8000.

### Port map

80/tcp   nginx       public — landing, library proxy, status proxy
8080/tcp kiwix-serve localhost only — ZIM hosting
8000/tcp uvicorn     localhost only — FastAPI /status
53/udp   dnsmasq     public — wildcard DNS
67/udp   dnsmasq     public — DHCP

# 3. Hardware Bill of Materials

Two profiles. The Zero 2 W build is the gift target: cheaper, lower power, fits in a small Pelican-style case. The Pi 4 build is for multi-user scenarios (community center, shelter).

| Component | Recommended part | Profile |
| --- | --- | --- |
| SBC | Raspberry Pi Zero 2 W | Gift / solo |
| SBC | Raspberry Pi 4 (4GB) | Community |
| Storage | Endurance microSD 128–256 GB (dashcam grade) | Both |
| Storage (alt) | USB 3 SSD 256–512 GB, rugged enclosure | Pi 4 only |
| Power | 10,000 mAh power bank w/ pass-through | Both |
| Solar (opt) | 10–20 W folding panel, USB out | Both |
| Cooling | Passive heatsink kit (no fans) | Both |
| Cable | Short braided USB-A → micro/USB-C | Both |
| Case | Pelican-style 1020, foam-lined | Both |

# 4. Software Stack

- OS: Raspberry Pi OS Lite (64-bit, Bookworm). No desktop.

- AP daemon: hostapd configured for WPA2 or open mode (configurable).

- DHCP/DNS: dnsmasq with wildcard address=/#/192.168.4.1.

- Web server: nginx (proxies to Kiwix and FastAPI, serves static landing).

- Content server: kiwix-tools (kiwix-serve) for ZIM files.

- Status API: FastAPI + uvicorn, single endpoint /api/status returning JSON.

- Process supervision: systemd units for hostapd, dnsmasq, kiwix-serve, signal-status, nginx.

- Read-only root (optional v1.1): overlayfs to protect SD card from corruption on power loss.

# 5. Repository Layout

Single repo, MIT licensed. Claude Code will work primarily inside this tree:

coreconduit-signal/
├── README.md
├── LICENSE                       # MIT
├── install.sh                    # one-shot provisioner (idempotent)
├── uninstall.sh
├── Makefile                      # convenience targets
├── docs/
│   ├── ARCHITECTURE.md
│   ├── HARDWARE.md
│   ├── CONTENT_GUIDE.md          # how to source and load ZIMs
│   └── TROUBLESHOOTING.md
├── config/
│   ├── hostapd/hostapd.conf
│   ├── dnsmasq/signal.conf
│   ├── dhcpcd/signal.conf
│   ├── nginx/signal.conf
│   └── sysctl/signal.conf
├── systemd/
│   ├── signal-ap.service
│   ├── signal-kiwix.service
│   └── signal-status.service
├── web/
│   ├── public/
│   │   ├── index.html            # landing
│   │   ├── library.html
│   │   ├── status.html
│   │   ├── assets/css/signal.css
│   │   ├── assets/js/status.js
│   │   └── assets/img/
│   └── README.md
├── api/
│   ├── pyproject.toml
│   ├── signal_status/__init__.py
│   ├── signal_status/main.py     # FastAPI app
│   ├── signal_status/system.py   # battery, disk, uptime probes
│   └── tests/test_status.py
├── content/
│   ├── manifest.yaml             # ZIMs to fetch + sha256
│   ├── fetch.sh                  # downloads ZIMs to /var/lib/kiwix
│   └── verify.sh
└── scripts/
    ├── bake_image.sh             # produce flashable .img
    ├── healthcheck.sh
    └── factory_reset.sh

# 6. Phased Build Plan

Five phases. Each ends with a tagged commit, an acceptance test, and a `docs/PHASE_N.md` summary. Claude Code should not advance phases until acceptance criteria pass.

## Phase 1 — Bare AP

Goal: a Pi that broadcasts an SSID and hands out IPs. No web yet.

### Tasks

- Scaffold the repo (Section 5).

- Write `config/hostapd/hostapd.conf` (channel 6, SSID `SIGNAL_INFOHUB`, country code configurable).

- Write `config/dnsmasq/signal.conf` (DHCP 192.168.4.10–192.168.4.250, lease 12h, no wildcard yet).

- Write `config/dhcpcd/signal.conf` to give `wlan0` a static IP 192.168.4.1.

- Author idempotent `install.sh` step that installs packages and links configs.

- Add `systemd/signal-ap.service` that orders hostapd → dnsmasq.

### Acceptance

- From a phone: SSID appears, joining succeeds, phone receives an IP in 192.168.4.0/24.

- `systemctl status signal-ap` is green after a cold boot.

## Phase 2 — Captive portal redirection

Goal: every URL a client tries to reach is intercepted and answered by the Pi.

### Tasks

- Add wildcard DNS to dnsmasq (`address=/#/192.168.4.1`).

- Add nginx config that returns a 302 to `http://hub.local/` for the well-known captive-portal probe URLs (Apple, Android, Microsoft, Firefox).

- Default `server` block serves the landing page on `hub.local` and on any other Host header.

- iptables rule: REJECT outbound traffic from `wlan0` except to the Pi itself.

### Acceptance

- iOS, Android, and a laptop all auto-pop the captive-portal sheet on join.

- Typing any URL in the browser lands on the SIGNAL landing page.

## Phase 3 — Content layer

Goal: Kiwix is serving real ZIMs and is reachable through nginx.

### Tasks

- Author `content/manifest.yaml` with ZIM names, URLs, and sha256 hashes.

- Author `content/fetch.sh` (run on a workstation, not the Pi) and `verify.sh`.

- Install `kiwix-tools`; configure `signal-kiwix.service` to serve `/var/lib/kiwix/*.zim` on `127.0.0.1:8080`.

- nginx `location /library/` proxies to kiwix-serve, stripping the prefix.

- Document recommended starter payload in `docs/CONTENT_GUIDE.md` (Wikipedia no-images, WikiHow, Wiktionary, Project Gutenberg, OpenStreetMap regional).

### Acceptance

- `curl http://hub.local/library/` returns the Kiwix index.

- Browsing a Wikipedia article on a phone works without internet.

## Phase 4 — Frontend

Goal: a clean, mobile-first landing page in CoreConduit visual language.

### Tasks

- Static `index.html` with category tiles: Encyclopedia, How-To, Medicine, Maps, Library, Status.

- CSS uses the CoreConduit silver theme: navy header, silver surface, blue and orange accents, 3px gradient accent bar on cards.

- Typography: Exo 2 (display), Plus Jakarta Sans (body), IBM Plex Mono (code) — bundled locally, no CDN.

- Tool-name styling: orange second half (e.g. `Sig` + `nal`, `Info` + `Hub`).

- High-contrast accessibility mode toggle (`prefers-contrast: more` and a manual switch).

- `status.html` consumes `/api/status` and shows uptime, free storage, voltage, connected clients.

### Acceptance

- Lighthouse mobile score ≥ 95 for performance and accessibility (offline, served from the Pi).

- Page renders without JS for category navigation; status is the only JS-dependent view.

## Phase 5 — Status API & polish

Goal: device knows its own health and exposes it; install is one command from a fresh image.

### Tasks

- FastAPI app `signal_status` with a single `/api/status` GET returning JSON.

- Probes: uptime, load, free disk on `/var/lib/kiwix`, connected DHCP leases count, undervoltage flag from `vcgencmd get_throttled`.

- `signal-status.service` runs uvicorn bound to `127.0.0.1:8000`.

- `scripts/bake_image.sh` produces a flashable `.img.xz` ready for Raspberry Pi Imager.

- `scripts/factory_reset.sh` wipes user data but preserves ZIM library.

- ASCII-art SSH MOTD on login (the "tactical" boot screen).

### Acceptance

- Fresh Pi OS Lite image + `curl ... | bash install.sh` results in a working hub in under 15 minutes.

- `/api/status` returns valid JSON within 200ms; status page reflects pulled values.

# 7. How Claude Code Should Work This Plan

### Operating mode

- Treat each phase as an isolated PR. Don't bleed phase 3 work into phase 2.

- Run on a dev machine, not on the Pi. Use a Pi or a Pi emulator (QEMU + raspi-os) for integration testing.

- Every config file gets a header comment with: purpose, owning systemd unit, and which phase introduced it.

- All shell scripts: `set -euo pipefail`, `shellcheck` clean, no `curl | bash` inside scripts.

- Python: ruff + black + mypy clean. Tests with pytest.

- Commits: Conventional Commits (`feat:`, `fix:`, `docs:`, `chore:`).

### Required guardrails

- No silent network calls. Any outbound HTTP must be in `content/fetch.sh` only, and gated by an explicit user invocation.

- No secrets in repo. WPA2 passphrase is generated on first boot, written to `/boot/SIGNAL_WIFI.txt`, then removed from logs.

- Idempotent install. Re-running `install.sh` on a configured Pi must converge, not break.

### Definition of done (per phase)

- Acceptance tests pass on a physical Pi Zero 2 W.

- `docs/PHASE_N.md` documents what was built and how to verify.

- CHANGELOG entry added.

- Phase tag pushed: `v0.1.0-phase1`, etc.

# 8. Risks & Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| SD card corruption on power loss | Device bricks | Read-only root (v1.1); endurance SD; warn in docs |
| Captive-portal detection varies by OS/version | User sees no auto-popup | Test matrix: iOS 17/18, Android 13/14, Win 11, Fedora; document fallback URL |
| Wi-Fi chip overheats under load | AP drops | Passive heatsink; thermal throttling exposed in /api/status |
| ZIM file licensing varies | Distribution issues | Manifest tracks source + license per ZIM; CONTENT_GUIDE makes user responsible for their payload |
| Legal: "emergency device" implications | User overreliance | Landing page disclaimer: this is a knowledge cache, not a substitute for emergency services |
| Pi 5 vs Zero 2 W power profile drift | Install fails on one variant | CI matrix tests both; install.sh detects SoC and adjusts |

# 9. Open Questions for Cory

- WPA2 with a printed passphrase, or open SSID with a captive-portal terms page? (Open is friendlier in a true emergency; WPA2 is safer if the device is deployed long-term.)

- Should the v1.0 landing page include a CoreConduit footer and link back to the open-source repo, or stay completely neutral to avoid "branding in an emergency" awkwardness?

- Default payload: ship with empty `/var/lib/kiwix` and a fetch script, or pre-bake a starter image? Pre-baking is a much bigger artifact (~30+ GB).

- Read-only root in v1.0 or defer to v1.1? Adds complexity but materially improves SD card survival.

- Solar-charge-aware status (detect charging vs discharging) — worth the extra hardware (INA219 over I²C) or skip for v1.0?

# 10. Suggested Milestones

Rough sizing for a part-time pace. Adjust based on hardware availability.

- Week 1: Phase 1 (Bare AP) + repo scaffolding.

- Week 2: Phase 2 (Captive portal) + cross-OS detection testing.

- Week 3: Phase 3 (Content) + content manifest finalized.

- Week 4: Phase 4 (Frontend) + accessibility pass.

- Week 5: Phase 5 (Status & image bake) + end-to-end test on Zero 2 W.

- Week 6: Documentation polish, blog post draft, gift-packaging photos.

Note  This pairs naturally with a future CoreConduit blog post — "Small Models, Big Jobs" is already queued, and SIGNAL is a fitting concrete example of fit-for-purpose infrastructure for mission-driven contexts.